# Fabrication and Integration Process

### Material Choice Justification :

### MDF (5mm) for Stage Structure

- Chosen for its rigidity and structural stability, making it suitable for supporting motors and components

- Easy to cut, drill, and assemble, which speeds up fabrication

- Provides a smooth, uniform surface for painting and finishing

### PLA (3D Printing) for Gaming Console

- Lightweight, reducing overall system load

- Easy to 3D print with high accuracy, ideal for custom shapes

- Safer and more practical compared to resin (no toxic handling, easier post-processing)

- Good surface finish for prototyping and user interaction

2. Fabrication Steps :

### Stage Fabrication

- Measured and designed the stage layout based on component placement

- Laser cut 5mm MDF sheets into required shapes for base and walls

- Assembled the stage structure using joints and adhesives

- Created two rectangular T-joint boxes on top of the stage

- These boxes were designed to allow easy placement and access to circuits

- The design was made first according to 3mm which later upon realization I made a new file as per 5mm thickness of the board to make the box stand properly

- The idea behind the box was to have two rectangular on top to put the circuits in covered by side panels which gives it a stage feel

### Console Fabrication

- Designed the gaming console enclosure digitally

- 3D printed the console using PLA

- Cleaned and finished the printed parts for better aesthetics

- Mounted push buttons and OLED display into the console