# Electronics Documentation

### Electronics Block Diagram

Oled Circuit Diagram (Console)

Doll Servos Circuit Diagram

### Console Circuit - OLED & Buttons

The console ESP32 manages input, display, and communication through a continuous loop.

| Component | ESP32 Pin | Notes |
| --- | --- | --- |
| OLED SCL (SSD1306) | GPIO 22 | I2C Clock |
| OLED SDA (SSD1306) | GPIO 23 | I2C Data |
| Right Arrow Button | GPIO 27 | INPUT_PULLUP |
| Down Arrow Button | GPIO 14 | INPUT_PULLUP |
| Up Arrow Button | GPIO 32 | INPUT_PULLUP |
| Left Arrow Button | GPIO 13 | INPUT_PULLUP |
| Start Button | GPIO 25 | INPUT_PULLUP |
| Stop Button | GPIO 26 | INPUT_PULLUP |

### 2. Components Table

| Component | Quantity | Purpose | Justification |
| --- | --- | --- | --- |
| ESP32 | 2 | Main controller for console and stage | Supports wireless (ESP-NOW), GPIO control, and flexible coding |
| 1.3" OLED Display | 1 | Display game interface | Compact, low power, suitable for real-time visuals |
| Arrow Push Buttons | 4 | User input for gameplay | Simple, responsive directional control |
| Start/Stop Buttons | 2 | Control game flow | Improves usability and interaction |
| 18650 Li-ion Battery Pack | 1 | Power supply. | Portable and sufficient capacity for system |
| ESP-NOW Wireless (Sender/Receiver) | 2 | Communication between systems | Enables low-latency wireless data transfer |
| PCA9685 PWM Driver | 1 | Control multiple servos | Provides stable PWM signals beyond ESP32 limits |
| Servo Motors | 4 | Doll movement. | Easy angle control for directional motion. |
| Motor Power Module | 1 | Power distribution | Ensures adequate current for servos |
| Buzzer (Optional) | 1 | Feedback sound. | Enhances user interaction and feedback |