# Software Documentation

### 1. Software Overview

What does the software do?
The software controls the interactive game system and (initially) the communication between the console and the dancing doll. It reads button inputs, generates arrow-based gameplay on the OLED display, tracks score and errors, and triggers animations based on user input. A separate module was designed to send and receive movement commands to control the doll.

Platforms used

- MicroPython on ESP32

- Libraries: GPIO (Pin), I2C, PWM, SSD1306/SH1106, ESP-NOW

### 2. System Logic

- User presses Start button

- Game begins on OLED screen

- Arrows fall toward a hit line

- User presses corresponding direction buttons

- System checks timing accuracy

- Score, combo, and errors are updated

- Animation feedback is shown on screen

- (Planned) Signal sent via ESP-NOW to second ESP32

- (Planned) Doll performs corresponding movement

Final system: Steps 8–9 were not integrated; console and doll function independently

### Core Game Logic

- Notes are generated with timestamps and random directions

- A debounce function prevents false button triggers

- Game state includes score, combo, errors, and animation state

- Start/Stop buttons control the game loop

### Main Game Loop

- Continuously updates display and reads inputs

- Arrows move toward a fixed hit line

- Button presses are checked only on new input (not hold)

- If timing is correct → score increases

- If missed → error is recorded

- Animation changes based on input (idle, direction, error)

### ESP-NOW Sender (Console)

- Sends directional commands (UP, DOWN, LEFT, RIGHT)

- Avoids repeated signals using last_sent logic

- Designed to reduce communication overload

### ESP-NOW Receiver & Servo Control (Stage)

- Receives commands and activates corresponding servo

- Each movement is short and returns to neutral position

- Ensures controlled and repeatable motion

### 6.7 Alternate Servo Logic (Fallback System)

- Direct button-to-servo mapping without ESP-NOW

- Each press increases servo angle cyclically

- Buzzer provides feedback

- Used as a working fallback when integration failed

### Final Insight

The software successfully delivered a responsive game system and modular control logic. While full wireless integration was not achieved, the project demonstrates strong understanding of input handling, real-time feedback systems, and hardware-software interaction.

### 3. Flowchart